Asbestos Awareness in the plumbing industry

[Text on screen: ASBESTOS THE HIDDEN KILLER]


KEVIN SHEEDY – JEMER AFL PLAYER & COACH: When I was a plumber back in the 60s and 70s, I didn't have a clue how dangerous asbestos was. None of my plumber or electrician mates did either. It was everywhere — in factories, office blocks, and two out of every three houses built between World War Two and the 1980s, and it's still in them now.

I count myself lucky. I'm not one of those thousands of Australian traders who have died or had their health ruined by this stuff. The terrible thing about asbestos is that it's a hidden killer. You can't smell it, you can't taste it, and you can't see the microscopic fibres that get into your lungs and cause the damage. 

That's why it's vital for every tradies — especially plumbers and electricians like you. To know the places where asbestos could be present. And here's the bottom-line: if you have any reason at all to believe there might be asbestos where you're working. Stop! Don't risk it, and don't let your mates risk it either. If you see something, say something. And remember, no one has the right to pressure you into working for asbestos might be present.

Okay, so where we find asbestos?  Unfortunately, it's hiding in lots of places and in lots of different forms. 

[Music]

[Text on screen: ASBESTOS CEMENT]

One of the most common is asbestos cement, which is the cement mixed with asbestos, and compressed into different shapes. It's a high grey material using corrugated sheets on the roofs of industrial buildings, garages, and sheds. It's also found in wall cladding, down pipes, and the gutters. 

[Music]

[Text on screen: LOOSE FILL ASBESTOS]

Loose fill asbestos was used to insulate in Tusculum domestic buildings, so it's found in wall cavities and under floorboards. It's usually blue grey or whitish in colour. And because it's light and fluffy is probably the most dangerous form of asbestos. 

[Music]

[Text on screen: ASBESTOS IN LAGGING]

Because of its insulating properties, you'll often find asbestos used as lagging around pipes, and water heaters, something for plumbers to especially look out for. This kind of asbestos can be hard to spot, because it's often painted. 

[Music]

[Text on screen: ASBESTOS IN ELECTRICAL INSULATION]

Electricians need to take special care with illegible switchboard panels and meter panels. Those installed before 1990, we're almost certainly have used asbestos as an insulator. And so will many that were built before 2004.

[Music]

[Text on screen: TEXTURED COATING ASBESTOS]

Asbestos was also used to provide a textured coating on ceilings and walls. Sometimes known under trade names like our cakes. This would originally have been white but has often been painted over, so the thing to watch out for is a decorative finish. 

[Music]

[Text on screen: ASBESTOS INSULATING BOARD]

Asbestos insulating board, or AIB, was commonly used for fireproofing, so it can be found in petition walls, fire doors, lift-shaft linings, ceiling tiles, and the panels below windows. Unfortunately, like a lot of asbestos, this dust can be hard to tell, apart from non-asbestos materials. So, if you're not sure —stop and find out! Don't chance it. Reporting it. 

[Music]

[Text on screen: OTHER PLACES TO LOOK]

Other places that this killer can be hiding are in asbestos floor tiles and carpet underlay. Another kind of asbestos— asbestos paper— was used the lining under tiles.

[Music]

[Text on screen: ASBESTOS]

Asbestos composites are also common around toilet cisterns and toilet seats. windows sills and bark panels. You should also keep an eye out for different kinds of asbestos in fire doors, street-light control boxes, trenching pit covers and cable trenching. As you can see, asbestos was once used just about everywhere, and it's still in a lot of places where you might be working. Sawing, hammering, standing, or drilling into it can send microfibres into the air and into your lungs. So let me say it again: if you have any reason to think there might be a specialist where you're working, stop. And if there's any problem, your union will always back you up.

[Music]

EARL SETCHES – FEDERAL SECRETARY CEPU PLUMBING DIVISION: No one has the right to make you work in unsafe conditions. No one. There are some basic steps you can take before starting any job. First, you can ask if there’s a hazardous substances register for the site. You can also directly ask if there’s any asbestos on the site. And if you have any reason to suspect there might be any asbestos, stop. Don’t risk it. Someone has to prove it’s not asbestos; until then, it’s considered to be asbestos.

ALLEN HICKS – ETU NATIONAL SECRETARY: If you think there may be asbestos in your workplace, speak out. Talk to your workmates about it, talk to your union delegate or safety rep, talk to your boss. Whatever you do, don’t just go ahead and do the job anyway. You can also call the safety regulator in your state, and you don’t have to give them your name.

EARL SETCHES: And don’t worry that you might be wrong. Your life and your future health always come first.

KEVIN SHEEDY: When I was a young bloke, I thought I could run through brick walls, but as I found out all too often, I wasn’t invulnerable — and neither are you. So never, ever take risks with asbestos. Just when you should be earning a good income as an electrician or a plumber, enjoying life, raising a family, you could find yourself hooked up to an oxygen tank — no job, no future, just slowly dying. And I’m painting a pretty ugly picture, but that’s the way it is.

So have another look at this DVD and take a closer look at the different kinds of asbestos and where you might come across them. And let me say it one more time: if you ever have any reason at all to think there might be asbestos where you’re working, stop. Don’t risk it — and don’t let your mates risk it either.

[Text on screen: ASBESTOS. THE HIDDEN KILLER.]

[Music]

[Image: ASSEA logo]